Manufacturability Screening

Automated Machining Feature Recognition from CAD Geometry

Vishal Prabu  |  October 2026

# Project Overview

This project is a proof of concept demonstrating that, given a CAD part, an AI system can automatically recognize which machined feature type each face belongs to, such as a pocket, slot, or chamfer, directly from its geometry. It is a self-directed technical exploration and a companion effort to a related design-reuse project, both aligned with the Industrials team's broader direction on applying GenAI to the engineering design lifecycle. The proof of concept now also analyzes any STEP file, summarizes the recognized features for the whole part, applies rule-based manufacturability flags, and provides a simple web interface.

# Objectives

- Establish whether the geometric deep learning technique used for design-reuse search can be adapted, via transfer learning, to a different task: automated machining-feature recognition.

- Build and validate an end-to-end proof of concept trained on real, labeled CAD geometry.

- Lay the technical groundwork for a manufacturability screening capability, where recognized features are checked against manufacturing rules to flag design-for-manufacturing concerns.

- Make the capability usable on any STEP file through a command-line tool and a simple web interface.

# Why This Matters

Manufacturability issues, such as deep narrow pockets, sharp internal corners, or features that are hard to reach with standard tooling, are often only caught late in the design cycle, during manual review or after a part reaches a machinist. Automatically recognizing machining features from geometry is the foundational step toward flagging these concerns earlier, while a design is still being developed. This capability is a natural extension of the geometric design-reuse work explored in parallel, and reflects the same broader direction: applying GenAI to specific, well-defined problems in the engineering design lifecycle rather than a generic overview of what AI can do.

# Technical Approach

The system is built through transfer learning: reusing a pretrained geometric deep learning model rather than training one from scratch.

- Backbone (BRepNet): a deep learning model pretrained to understand CAD geometry directly from B-Rep topology (faces, edges, surfaces), reused unchanged from the design-reuse project. Its parameters are kept frozen; only a new classification layer is trained on top.

- Training data (MFCAD): an open-source dataset of 15,488 CAD parts labeled with 16 machining feature classes (pockets, slots, steps, chamfers, and others). A verified 1,000-part subset was used for training.

- Class-weighted training: one feature class, unmachined stock surface, makes up nearly a third of all labeled faces. A naive training approach collapsed onto predicting that majority class for most parts. Reweighting the training loss by class frequency resolved this: every one of the 16 classes is now recognized with some accuracy, rather than several being ignored entirely.

- Correctness safeguards: face labels are matched by the face names embedded in each STEP file (verified on all 1,000 parts), the frozen backbone is kept in evaluation mode so its normalization statistics cannot drift, and the best validation checkpoint is kept because accuracy fluctuates between epochs.

# Capabilities

- New-part analysis: any STEP file is processed in an isolated subprocess and every face is classified with a confidence score. Unreadable or malformed files return an error instead of crashing the tool.

- Part summary: for each recognized class, the face count, an approximate feature instance count (adjacent faces of the same class), the mean confidence, and the model reliability for that class (low, medium or high, from its validation recall).

- Manufacturability flags: recognized feature types map to review items such as blind-depth control, internal corner radius, acute or multi-sided profiles, and non-orthogonal setups. Flags for low-reliability classes are worded as possible rather than detected.

- Web interface (Streamlit): an interactive 3D view colored by predicted or true feature class, with low-confidence faces greyed out, alongside the feature summary and the flags grouped by severity.

# Results to Date

- Achieved 60.4% face-level classification accuracy (2,694 of 4,464 faces) across all 16 machining feature classes on held-out validation data, up from a collapsed result in which 8 of 16 classes were never correctly predicted, before class-weighted training was introduced.

- The largest gains came from class-weighted training, longer training, and a small two-layer classification head in place of a single linear layer, using a 1,000-part training subset.

- Accuracy varies meaningfully by feature type and by part. Some classes, such as rectangular through-slots and stock surface, exceed 80% accuracy; more geometrically ambiguous or underrepresented classes, such as rectangular blind steps, remain considerably weaker.

- The new-part analysis path reproduces the 60.4% validation result exactly, confirming that predictions on new files are processed identically to training. Analysis takes about 5 seconds per part, including model loading.

- On sample validation parts, the flags raised from predictions were a subset of those raised from the true labels, with no false alarms.

- This stage demonstrates that the transfer-learning technique itself works and generalizes across the full feature vocabulary. Current results are proof-of-concept grade, not yet at the accuracy level required for a production manufacturability screening tool.

# Example Classification Results

Example 1: per-face detail for one test part. Predictions in red indicate faces the model classified incorrectly.

| Face | True class | Predicted class | Result |
| --- | --- | --- | --- |
| 0 | stock | rectangular_blind_step | Wrong |
| 1 | stock | stock | Correct |
| 2 | rectangular_through_slot | rectangular_through_slot | Correct |
| 3 | stock | 6sides_passage | Wrong |
| 4 | stock | 6sides_passage | Wrong |
| 5 | rectangular_through_slot | rectangular_through_slot | Correct |
| 6 | stock | stock | Correct |
| 7 | rectangular_passage | rectangular_passage | Correct |
| 8 | rectangular_passage | rectangular_passage | Correct |
| 9 | rectangular_passage | rectangular_passage | Correct |
| 10 | rectangular_passage | rectangular_passage | Correct |
| 11 | rectangular_through_slot | rectangular_through_slot | Correct |
| 12 | stock | stock | Correct |

Example 2: accuracy spot-checked across additional test parts, showing typical variation by part complexity.

| Test part | Face-level accuracy |
| --- | --- |
| Test part 0-0-0-2-19 | 76.9% (10 / 13 faces) |
| Test part 0-0-2-7-7-23 | 61.1% (11 / 18 faces) |
| Test part 0-7-7-10-13-23 | 40.0% (8 / 20 faces) |

# Known Limitations

- Flags are indicative only: they are derived from recognized feature types, with no dimensional checks on depth, width or corner radius.

- Flags fire sparsely on predictions, because a confidence threshold suppresses classes the model is less certain about.

- The model was trained only on MFCAD parts. The feature extractor requires a single closed solid, and other geometry has not been tested.

# Value for Engineers

- Automatically labels machining features on a CAD part without manual annotation, a foundational step for downstream manufacturability checks.

- Paired with rule-based checks, recognized features already produce indicative design-for-manufacturability flags, such as blind features that need depth control or non-orthogonal features that complicate machining setup.

- Reuses the same pretrained geometric backbone as the design-reuse project, so the technical investment compounds across use cases rather than starting over each time.

- Trained and validated on real CAD geometry via the same STEP-based approach used across this GenAI-for-engineering-design effort, keeping it broadly portable across CAD systems.

# Next Steps

- Add dimensional rules measured from the faces of each recognized feature, such as depth-to-width ratio and corner radius, to make flags quantitative.

- Improve classification accuracy further, most likely by training on a larger portion of the available 15,488-part dataset and by fine-tuning part of the backbone rather than only the classification layer.

- Use recognized features to filter or rank results in the related design similarity search.

- Explore generating natural-language explanations of flagged manufacturability concerns, for a more accessible, conversational interface.

- Assess feasibility on a specific client's CAD data and feature vocabulary.